Vernier Sensor Block
for LEGO Mindstorms EV3

Vernier has created a block for the LEGO Mindstorms EV3 software that allows the EV3 to read and display values from a wide variety of Vernier sensors when used with the Vernier NXT Sensor Adapter, (order code: BTA-NXT).

The Vernier Sensor Block allows you to use a subset of Vernier sensors to measure or compare in the same way you would use LEGO’s own sensors. Sensor calibrations are loaded automatically, and readings are returned as numerical values, in proper sensor units.

The block provides an "Oversampling" option. This option takes a large number of samples and returns the averaged reading. Turn this "On" if you would like your sensor readings to be less erratic. Note that oversampling slows down how quickly readings are returned (about one every half-second). This is a good option to choose if you are displaying the reading on the EV3 brick's screen, or if you are using a sensor, such as the differential voltage probe or pH, that benefits from a slower, more stable output.

Installation

To use Vernier sensors with the EV3, you will need to import the Vernier Sensor Block.

- Launch MINDSTORMS EV3 software and open a new programming project.

- Choose Block Import from the Tools menu.

- Click Browse to find and open the Vernier ev3b block from the download.

- Click the Vernier EV3 Block entry in the "Select Blocks to Import" section to highlight it.

- Click Import.

- Close the Import window when the software notifies you that the import is complete and successful.

- Restart the EV3 software for the change to take effect.

- The block is ready for use.

Examples

Open the examples found in the download to get started using Vernier sensors with the EV3.